Bluetooth Adapter Recommendations

Joel and Gemini

03/15/2026 11:51:25

I've updated the workflow to ensure the Panda (CSR) driver and the new Intel driver don't get into a "resources war."

As an engineer, you know that keeping the software stack clean is half the battle. Installing the Intel drivers now is actually proactive—it populates the Windows Driver Store so that the moment the hardware is sensed, the handshake happens automatically.

# Revised "Clean Swap" Workflow

## Phase 1: Software Prep (Now)

- Install Intel Drivers: Run the Wi-Fi and Bluetooth installers you downloaded. They will "stage" themselves in the system.

- Unpair Devices: Go to your Bluetooth settings and Remove any devices currently paired to the Panda dongle.

- Disable Panda:

Unplug the Panda USB dongle.

Go to Device Manager > View > Show hidden devices.

Find any "CSR Bluetooth" or "Panda" entries under Bluetooth or Universal Serial Bus controllers.

Right-click and select Uninstall device (check the box for "Attempt to remove the driver" if prompted).

## Phase 2: Hardware Swap (When the eBay card arrives)

- Power Down: Shut down the E5570 and unplug the AC adapter.

- Internal Safety: Remove the back cover and disconnect the battery cable from the motherboard (standard practice to avoid ESD or shorting the M.2 pins).

- The Swap: * Unscrew the metal bracket and the single M2 screw holding the 8260.

Slide the 8260 out; slide the AX210 in.

Snap the Black (Main) and Grey (Aux) antennas onto the new card.

- Reassemble: Reconnect the battery, replace the cover, and power up.

## Phase 3: Post-Install Verification

- Windows 10: Open Device Manager. You should see Intel(R) Wi-Fi 6E AX210. If Bluetooth doesn't appear, remember to check that F2 BIOS "Wireless Device Enable" menu we verified earlier.

- Ubuntu: Open a terminal and run:

lsusb |

grep

-i bluetooth

Note: Even though it’s an M.2 card, the Bluetooth portion will show up on the USB bus.

# One Final EE Check

Since your BIOS already has "Bluetooth" enabled, the motherboard is ready to provide the 3.3V and USB data lines to that slot. The AX210 will take it from there.

Would you like me to find the specific terminal commands to permanently "blacklist" the old Panda/CSR kernel modules in Ubuntu so they don't interfere with the new Intel ones?

yes plz do find blacklist commands

To avoid any "ghost" signals from the old Panda dongle interfering with your new AX210 in

Ubuntu, it's wise to blacklist its specific driver. The Panda adapter uses a CSR (Cambridge Silicon Radio) chipset, which typically relies on the btusb driver—the same driver the AX210 will use.

Because of this, we don't want to blacklist btusb entirely, or your new card won't work either. Instead, the best approach for an engineer is to unplug the dongle and then tell the kernel to ignore that specific USB ID if it's ever plugged back in.

# Ubuntu Blacklist Procedure

## 1. Identify the "Old" Driver

With the Panda dongle plugged in, run this in your terminal to see exactly which module is claiming it:

lsmod |

grep

bt

You will likely see

btusb

. If you see

bnep

or

rfcomm

, those are also part of the stack.

## 2. Create a Surgical Blacklist

Since we can't blacklist the whole btusb module (the AX210 needs it), the most effective way to "silence" the Panda is to use an install /bin/false directive for its specific hardware ID.

- Create a new configuration file:

sudo nano /etc/modprobe.d/blacklist-panda.conf

- Add the following line (this effectively "kills" the driver's ability to load for CSR-based devices without affecting the Intel ones):

# Prevent CSR-based Panda dongle from loading

install btusb /sbin/modprobe --ignore-install btusb && { echo "0a12 0001" > /sys/bus/usb/drivers/btusb/remove_id; }

Note: 0a12 0001 is the standard hardware ID for those CSR 4.0 Panda dongles.

## 3. Update Initramfs

To ensure these changes stick through the boot process:

sudo

update

-

initramfs

-

u

# The "Double-Clean" Workflow

Step

Action

Why?

1

Unplug Panda

Prevents physical hardware conflicts during the handshake.

2

Install AX210

Drivers

Populates the "Driver Store" (Windows) and kernel modules (Ubuntu).

3

Blacklist Panda ID

Ensures Ubuntu doesn't try to use the "sad panda" drivers if you

accidentally plug it back in.

4

Physical Swap

Replaces the 8260 with the AX210.

# One Final Engineering Tip

In Ubuntu, if you notice your Bluetooth is "Hard Blocked" after the swap, don't panic. Run:

rfkill unblock bluetooth

This is the standard command to tell the kernel that the "software kill switch" is off.

Would you like me to find the specific udev rule to prioritize the internal AX210 over any other Bluetooth device that might be plugged in? This ensures your BLE sensors always point to the strongest radio.