# Cardiac Autonomic Markers in LF/HF and Time Domains

Journal-Style Variant

Prepared from source document dated 03/17/2026

## Abstract

Heart Rate Variability (HRV) frequency-domain metrics (LF, HF, and LF/HF) and time-domain metrics (RMSSD, SDNN) are useful but context-sensitive indicators of autonomic regulation. HF (0.15-0.40 Hz) is predominantly vagal and respiration-linked, LF (0.04-0.15 Hz) reflects mixed autonomic influence with substantial baroreflex contribution, and LF/HF is best interpreted as a contextual composite rather than a branch-specific index. Reliable trend analysis requires standardized acquisition, artifact control, and baseline-referenced interpretation.

## 1. Background

HRV captures beat-to-beat R-R interval variability. Spectral decomposition via PSD estimates the distribution of variance across physiological frequency bands, offering a practical non-invasive window into autonomic control dynamics.

## 2. Frequency-Domain Interpretation

HF (0.15-0.40 Hz) is primarily associated with parasympathetic modulation and RSA.

LF (0.04-0.15 Hz) includes mixed autonomic influences and is strongly shaped by baroreflex mechanisms.

LF/HF should not be used as a direct sympathetic-to-parasympathetic ratio in isolation.

PHF = 0.150.40S(f) df

nHF = HFTotal Power - VLF × 100

Table 1: Frequency-domain definitions and interpretive cautions.

| Metric | Range / Formula | Physiological Weighting | Interpretive Note |
| --- | --- | --- | --- |
| HF | 0.15-0.40 Hz | Predominantly vagal | Recovery-sensitive when measured under stable conditions |
| LF | 0.04-0.15 Hz | Mixed ANS + baroreflex | Not purely sympathetic |
| LF/HF | LF / HF | Composite | Contextual marker, not branch-isolation |

## 3. Trend Acquisition Protocol

Use 3-5 minute recordings immediately upon waking, before caffeine, with fixed posture.

Prefer rolling baselines (for example, 7-day means) over isolated values.

Capture major confounders (sleep, illness, hydration, stimulant timing, training load).

Control breathing cadence during comparisons, as respiration shifts can alter spectral allocation.

## 4. Time-Domain Metrics and Operational Thresholds

RMSSD is the primary short-window vagal proxy; SDNN reflects broader total variability with duration dependence.

Baseline window: approximately 14 days.

Amber condition: RMSSD decrease of about 0.5-1.0 SD below baseline.

Red condition: RMSSD decrease greater than 1.5 SD below baseline.

## 5. Electrotherapy Monitoring Considerations

Short transient RMSSD decreases at stimulation onset can be expected.

Persistent downward RMSSD drift may indicate excessive stimulation burden.

Electrotherapy-related electrical interference can corrupt ECG/PPG beat detection.

Artifact-free recordings should be prioritized over longer noisy recordings.

Table 2: Practical metric selection in short-session monitoring.

| Metric | Preferred Use | Primary Caveat |
| --- | --- | --- |
| RMSSD | Short-session bio-safety and recovery trend tracking | Artifact sensitivity |
| SDNN | Longer-duration total variability context | Weak in short windows |
| LF/HF | Supplementary frequency-domain context | Oversimplified if used alone |

## 6. Integrated Interpretation Guidance

Use metric pairs (for example, RMSSD + resting heart rate or HF + total power) for better contextual decisions.

Interpret thresholds as individualized baseline deviations, not fixed universal cutoffs.

Perform occasional longer standardized sessions to verify that short-form trends remain calibrated.

## Conclusion

Robust HRV interpretation requires combining physiological theory with disciplined measurement practice. When acquisition is standardized and artifact control is explicit, frequency- and time-domain metrics provide actionable and complementary insight into autonomic state.